LASC Level 1 Compliance Tests

Conformance Process for Specification and Acceptance Testing of LASC Communication Protocol

| VERSION: [1.0] | REVISION DATE: [2/10/09] |
| --- | --- |

| Approver Name | Title | Signature | Date |
| --- | --- | --- | --- |

Table of Contents

1.	Introduction	3

2.	Testing Encoding	3

3.	Testing Details	3

# Introduction

This testing regime covers the encoding of complete testing of LASC Level1 Compliance. This covers, in brief, the EIP stack (Layers 1,2,3/4) and a subset of the Application layer functionality (Layer 7). This is the full communication channel between LASC compliant modules.

# Testing Encoding

The testing details are discussed fully in document LASC Level 1 Specification. The XML representation for the tests is described in the document Technical Specifications.

# Testing Details

For traditional testing, each test should provide:

- Component identification, including a short descriptive name

- Pre-requisite tests

- Testing input requirements

- Test function

- Testing output requirements

- Fail Severity indication.

| Test case | Test pre-requisites | Input State/Requirements | Event | Output | Fail severity |
| --- | --- | --- | --- | --- | --- |
| - Base Ethernet Communications |  |  | Device Startup |  |  |
| - Test ping |  | Idle Device | Ping Device | Successful Ping | Critical |
| - Test UDP request / response | 4.1 | Idle Device | Send UDP packet & wait for response | Output from List Identity Command. Pass / Fail. | Critical |
| - Test TCP request / response | 4.1 | Idle Device | Send TCP packet & wait for response. | Output from List Identity Command. Pass / Fail. | Critical |
| - Concurrent TCP Connections |  |  |  |  |  |
| 4.4.a Open 16 Concurrent TCP Connections |  | Idle Device | Open 16 concurrent TCP connections | Will Return Fail if all 16 connections could not be opened | Critical |
| 4.4.b Test 16 Concurrent TCP Connections | 4.4.a | Idle Device | Test communications with the 16 opened TCP connections | Will return fail, if all 16 TCP don’t communicate as expected | Critical |
| 4.4.c Close 16 Concurrent TCP Connections | 4.4.a & 4.4.b | Idle Device | Close 16 concurrent TCP connections opened in 4.4.a | Will return fail, if all 16 TCP sessions don’t close properly. | Critical |
| - Ethernet cable removed directly from device for 2 seconds | 4.1 | Idle Device &Network switch connected directly between PC and device under test. | Establish connection and watch connection timeout | Connection should remain active. | Medium |
| - Ethernet cable removed directly from device for 120 seconds | 4.1 |  | Establish connection and watch connection timeout | Connection should have timed out after 120 seconds, and return connection fault. | Medium |
| - Ethernet cable removed directly from PC for 2 seconds | 4.1 | Idle Device & Network switch connected between PC and device under test. | Establish connection and watch connection timeout. | Connection should remain active. | Medium |
| - Ethernet cable removed directly from PC for 120 seconds | 4.1 |  | Establish connection and watch connection timeout. | Connection should have timed out after 30 seconds, and return connection fault. | Medium |
| - Testing Encapsulation Commands |  |  |  |  |  |
| - Test NOP |  |  |  | . |  |
| - Test NOP |  | No EIP Session registered. | A NOP message is sent. | Successful if nothing received. | High |
| - Test NOP | 5.1.1 | EIP session registered. |  | Successful if nothing received. | High |
| - Test ListIdentity TCP or UDP |  |  |  |  |  |
| - Test ListIdentity TCP |  | No EIP Session registered | ListIdentity message is sent via TCP. | Successful if DUT responds with valid ListIdentity response. | Medium |
| - Test ListIdentity TCP |  | EIP Session registered | ListIdentity message is sent via TCP. | Successful if DUT responds with valid ListIdentity response. | Medium |
| - Test ListIdentity UDP |  | No EIP Session | ListIdentity message is sent via UDP. | Successful if DUT responds with valid ListIdentity response. | Medium |
| - Test RegisterSession TCP |  |  |  |  |  |
| - Test RegisterSession TCP |  | No EIP Session registered. | RegisterSession message is sent via TCP | Successful if a valid response is received. | Critical |
| - Test RegisterSession TCP Protocol version 2. |  | No EIP Session registered. | RegisterSession message is sent via TCP with Protocol version 2. | Successful if valid response is received with status 0x69 and the highest supported version in the protocol version field. | Medium |
| - Test UnRegisterSession (TCP) |  |  |  |  |  |
| - Test UnRegisterSession |  | No EIP Session registered. | UnRegisterSession message is sent | Successful if no response is received. | High |
| - Test UnRegisterSession |  | EIP session registered. | UnRegisterSession message is sent | Successful if no response is received. | High |
| - Test ListServices (TCP or UDP) |  |  |  |  |  |
| - Test ListServices (TCP) |  | No EIP Session registered. | ListServices message is sent via TCP | Successful is valid response is received from DUT. | Medium |
| - Test ListServices (TCP) |  | EIP session registered. | ListServices message is sent via TCP | Successful if valid response is received from DUT. | Medium |
| - Test SendRRData TCP |  |  |  |  |  |
| - Test SendRRData |  | No EIP Session registered. | SendRRData message is sent via TCP with class:1, instance 1, service 14. | Successful if no message received from DUT | High |
| - Test SendRRData |  | EIP session registered. | SendRRData message is sent via TCP with class:1, instance 1, service 14. | Successful valid message is received from DUT | High |
| - Testing Session Handling / Message Handling |  |  |  |  |  |
| - Message Handling |  |  |  |  |  |
| - Send truncated message, test message timeout |  | EIP session registered. | Truncated test message is sent. | Successful if an error message is returned within 10 Seconds. | High |
| - Send long messages (dual message) |  | EIP session registered. | Send long message (dual message) | Successful if a correct response is received to each of the two messages. | Minor |
| - Send garbled message |  | EIP session registered. | Send garbled message (shorter than normal packet) | Successful if device returns an error. | Medium |
| - Send garbled message |  | EIP session registered. | Send garbled message (same size as normal packet) | Successful if device returns an error. | Medium |
| - Send garbled message |  | EIP session registered. | Send garbled message (longer than normal packet) | Successful if device returns an error. | Medium |
| - Send message flood |  | EIP session registered. | Send 1000 EIP messages with no delay between messages. | Successful if each a valid response is returned for each of the 1000 messages. | Medium |
| - Incorrect session identifier |  | EIP session registered. | Send messages with the wrong session identifier. | Successful if each message returns an error. | Critical |
| - Incorrect sender context |  | EIP session registered. | Send messages with negative/same sender context | Successful if returns sender context and returns no error. | Minor |
| - Incorrect session handle |  | EIP session registered. | Send messages with previous (and closed) session handle | Successful if error message 100 is returned. (same as 6.3.7) | High |
| - NOP interference test |  | No EIP Session registered. | NOP is sent, session is registered, NOP is sent, ListServices is sent, NOP is sent, UnRegisterSession. | Successful if NOP does not interfere with normal operation. | Medium |
| - Testing Implementation of common services. |  |  |  |  |  |
| - Test Get Attribute single service |  | EIP session registered. |  |  | Critical |
| - Testing Implementation of Core Objects |  |  |  |  |  |
| - Test all EDS Attributes |  | EIP session registered | RRData message is sent for each attribute (class and instance) coded in the device EDS file. | Successful if all message respond with status 0 and the value returned is as coded in the EDS file | High |
| - Error Codes |  |  |  |  |  |
| - Invalid or unsupported encapsulation command |  | EIP session registered. | Send command 0xff | Successful if response is 0x1 | High |
| - Path destination unknown |  | EIP session registered. | Use class 0xff, instance 0xff, attribute 0xff | Successful if response is 0x5 | Medium |
| - Invalid session handle |  | EIP session registered. | Use incorrect session handle | Successful if response is 0x64 | High |
| - Testing Invalid Length Message |  | EIP session registered. | Use invalid length data | Successful if response is 0x65 | High |
| - unsupported encapsulation protocol version |  | EIP session registered. | Register session message with protocol version 55 | Successful if response is 0x69 | High |
| - Testing Timeouts |  |  |  |  |  |
| - List Identity Response |  |  |  | Response <250ms | High |
| - List Services Response |  |  |  | Response <250ms | High |
| - Unconnected Explicit Response |  |  |  | Response <250ms | High |
| - Testing Two Back to Back Explicit Message Requests |  |  |  | Response <250ms | High |
| - Retest concurrent sessions |  |  |  |  |  |
| - Open 16 Concurrent TCP Connections |  | Idle Device | Open 16 concurrent TCP connections | Will Return Fail if all 16 connections could not be opened | Critical |
| - Test 16 Concurrent TCP Connections |  | Idle Device | Test communications with the 16 opened TCP connections | Will return fail, if all 16 TCP don’t communicate as expected | Critical |
| - Close 16 Concurrent TCP Connections |  | Idle Device | Close 16 concurrent TCP connections opened in 11.1 | Will return fail, if all 16 TCP sessions don’t close properly. | Critical |

| Version | Description of Change | Author | Date |
| --- | --- | --- | --- |
| 1.0 | Created | Mark Dunn | 4/2/08 |