LASC Level 1 Compliance Tests – Landmark project completion version

Conformance Process for Specification and Acceptance Testing of LASC Communication Protocol

| VERSION: [1.0] | REVISION DATE: [2/10/09] |
| --- | --- |

| Approver Name | Title | Signature | Date |
| --- | --- | --- | --- |

Table of Contents

1.	Introduction	3

2.	Testing Encoding	3

3.	Testing Details	3

# Introduction

This testing regime covers the encoding of complete testing of LASC Level1 Compliance. This covers, in brief, the EIP responses required as at December 2007, ie, the end of Landmark project. This is a subset of LASC Level 1 testing.

# Testing Encoding

The testing layout will be discussed fully in document LASC Level 1 Specification – Communication protocol. This section gives an overview to enable creation of new tests.

# Testing Details

For traditional testing, each test should provide:

- Component identification, including a short descriptive name

- Pre-requisite tests

- Testing input requirements

- Test function

- Testing output requirements

- Fail Severity indication.

| Test case | Test pre-requisites | Input State/Requirements | Event | Output | Fail severity |
| --- | --- | --- | --- | --- | --- |
| - Base Ethernet Communications |  |  | Device Startup |  |  |
| - Test ping |  | Idle Device | Ping Device | Successful Ping | Critical |
| - Test TCP request / response | 4.1 | Idle Device | Send TCP packet & wait for response. | Output from List Identity Command. Pass / Fail. | Critical |
| - Concurrent TCP Connections |  |  |  |  |  |
| 4.4.a Open 16 Concurrent TCP Connections |  | Idle Device | Open 16 concurrent TCP connections | Will Return Fail if all 16 connections could not be opened | Critical |
| 4.4.b Test 16 Concurrent TCP Connections | 4.4.a | Idle Device | Test communications with the 16 opened TCP connections | Will return fail, if all 16 TCP don’t communicate as expected | Critical |
| 4.4.c Close 16 Concurrent TCP Connections | 4.4.a & 4.4.b | Idle Device | Close 16 concurrent TCP connections opened in 4.4.a | Will return fail, if all 16 TCP sessions don’t close properly. | Critical |
| - Ethernet cable removed directly from device for 2 seconds | 4.1 | Idle Device &Network switch connected directly between PC and device under test. | Establish connection and watch connection timeout | Connection should remain active. | Medium |
| - Ethernet cable removed directly from PC for 2 seconds | 4.1 | Idle Device & Network switch connected between PC and device under test. | Establish connection and watch connection timeout. | Connection should remain active. | Medium |
| - Ethernet cable removed directly from PC for 120 seconds | 4.1 |  | Establish connection and watch connection timeout. | Connection should have timed out after 30 seconds, and return connection fault. | Medium |
| - Testing Encapsulation Commands |  |  |  |  |  |
| - Test NOP |  |  |  | . |  |
| - Test NOP |  | No EIP Session registered. | A NOP message is sent. | Successful if nothing received. | High |
| - Test NOP | 5.1.1 | EIP session registered. |  | Successful if nothing received. | High |
| - Test ListIdentity TCP or UDP |  |  |  |  |  |
| - Test RegisterSession TCP |  |  |  |  |  |
| - Test RegisterSession TCP |  | No EIP Session registered. | RegisterSession message is sent via TCP | Successful if a valid response is received. | Critical |
| - Test RegisterSession TCP Protocol version 2. |  | No EIP Session registered. | RegisterSession message is sent via TCP with Protocol version 2. | Successful if valid response is received with status 0x69 and the highest supported version in the protocol version field. | Medium |
| - Test UnRegisterSession (TCP) |  |  |  |  |  |
| - Test UnRegisterSession |  | No EIP Session registered. | UnRegisterSession message is sent | Successful if no response is received. | High |
| - Test UnRegisterSession |  | EIP session registered. | UnRegisterSession message is sent | Successful if no response is received. | High |
| - Test ListServices (TCP or UDP) |  |  |  |  |  |
| - Test ListServices (TCP) |  | No EIP Session registered. | ListServices message is sent via TCP | Successful is valid response is received from DUT. | Medium |
| - Test ListServices (TCP) |  | EIP session registered. | ListServices message is sent via TCP | Successful if valid response is received from DUT. | Medium |
| - Test SendRRData TCP |  |  |  |  |  |
| - Test SendRRData |  | No EIP Session registered. | SendRRData message is sent via TCP with class:1, instance 1, service 14. | Successful if no message received from DUT | High |
| - Test SendRRData |  | EIP session registered. | SendRRData message is sent via TCP with class:1, instance 1, service 14. | Successful valid message is received from DUT | High |
| - Testing Session Handling / Message Handling |  |  |  |  |  |
| - Message Handling |  |  |  |  |  |
| - Send truncated message, test message timeout |  | EIP session registered. | Truncated test message is sent. | Successful if an error message is returned within 10 Seconds. | High |
| - Send garbled message |  | EIP session registered. | Send garbled message (same size as normal packet) | Successful if device returns an error. | Medium |
| - Testing Implementation of common services. |  |  |  |  |  |
| - Test Get Attribute single service |  | EIP session registered. |  |  | Critical |
| - Testing Implementation of Core Objects |  |  |  |  |  |
| - Test all EDS Attributes |  | EIP session registered | RRData message is sent for each attribute (class and instance) coded in the device EDS file. | Successful if all message respond with status 0 and the value returned is as coded in the EDS file | High |
| - Error Codes |  |  |  |  |  |
| - Invalid or unsupported encapsulation command |  | EIP session registered. | Send command 0xff | Successful if response is 0x1 | High |
| - Path destination unknown |  | EIP session registered. | Use class 0xff, instance 0xff, attribute 0xff | Successful if response is 0x5 | Medium |
| - Invalid session handle |  | EIP session registered. | Use incorrect session handle | Successful if response is 0x64 | High |
| - unsupported encapsulation protocol version |  | EIP session registered. | Register session message with protocol version 55 | Successful if response is 0x69 | High |
| - Testing Timeouts |  |  |  |  |  |
| - Retest concurrent sessions |  |  |  |  |  |
| - Open 16 Concurrent TCP Connections |  | Idle Device | Open 16 concurrent TCP connections | Will Return Fail if all 16 connections could not be opened | Critical |
| - Test 16 Concurrent TCP Connections |  | Idle Device | Test communications with the 16 opened TCP connections | Will return fail, if all 16 TCP don’t communicate as expected | Critical |
| - Close 16 Concurrent TCP Connections |  | Idle Device | Close 16 concurrent TCP connections opened in 11.1 | Will return fail, if all 16 TCP sessions don’t close properly. | Critical |

| Version | Description of Change | Author | Date |
| --- | --- | --- | --- |
| 1.0 | Created | Mark Dunn | 4/2/08 |