LASC Level 2 Compliance

Roof Support System Functional Testing

Conformance Process for Specification and Acceptance Testing of LASC Communication Protocol

| VERSION: [1.0] | REVISION DATE: [7/10/09] |
| --- | --- |

| Approver Name | Title | Signature | Date |
| --- | --- | --- | --- |

Table of Contents

1. Introduction	3

2 Functional Requirements	4

3 Communications Requirements	5

4 Testing Extension Details	6

5 Testing Details	6

# 1. Introduction

This testing regime covers the encoding of complete testing of LASC Level2 Compliance for a Roof Support System compliant device.

Design document ‘Interconnection of Landmark Compliant Longwall Mining Equipment – Roof Support System Communication and Functional Specification for Face Alignment.’ should be read in conjunction with this document.

The Longwall Roof Support system controls the advance of the armoured face conveyor (AFC) and is therefore the primary actuator in achieving controlled face alignment. From a control system perspective, the Roof Support system can be treated as a multi-channel position actuator with each channel corresponding to a roof support module and therefore a fixed point along the AFC. The shearer-mounted inertial navigation equipment is the primary position sensor and can be treated as a multi-channel position sensor with each channel corresponding to a fixed point along the AFC. The Landmark Controller processes information from the Shearer Position sensor and provides position correction information to the Roof Support system.

The full functional and communication requirements are specified below.

# 2 Functional Requirements

The general functional requirements of the Roof Support system actuator are as follows

F1. To advance the AFC at each Roof Support module a linear distance determined by summing the Roof Support Default Advance Distance (nominal web thickness) with the position correction information provided by the Landmark process controller.

F2. In addition to item 1, the Roof Support system is responsible for ensuring that the position correction information provided by the Landmark Controller cannot cause the Roof Support system to operate outside the designed control envelope.

F3. In the absence of valid position correction information, the Roof Support system is to assume a value of zero.

F4. All existing internal roof support control systems, including safety systems and timing and sequencing remain the responsibility of the Roof Support system.

F5. The Roof Support system is responsible for incorporating the position correction value into any automated manoeuvre algorithms (e.g. AFC snaking, gate-end turn-around) in such a way that the AFC position correction is achieved.

F6. To provide status information as requested by the Landmark Controller.

# 3 Communications Requirements

- The Roof Support system requests Recommended Position Correction values by setting an RPC Required flag (RSS Class 0x64, Attribute 9 – Status bit 0).

- This attribute is polled by the Landmark Controller and when this flag is set the Landmark Controller will supply the Roof Support system with an assembly of current RPC values and current Sequence Number

- The Roof Support system should clear the RPC Required flag as soon as the requested RPC assembly is received.

- The Roof Support system can determine the freshness of the RPC assembly by means of the sequence number.

- A unique (nominally sequential) sequence number will be assigned by the Landmark Controller at each point in the shear cycle when new Recommended Position Correction (RPC) values are available.

- A sequential number will be assigned to each shear half cycle. A shear half cycle is defined as a single complete traverse of the shearer across the longwall face.

- Sequence numbers will start at 0 and typically increment by 1 for each new set of RPC data supplied.

- When the sequence number reaches the maximum positive value allowable in an INT (32767), it will be reset to 0.

- A negative sequence number will indicate invalid data, an uninitialised state or an erroneous state

- A sequence number of –1 indicates that the Landmark system in un-initialised or in an unknown state. This is typically a Landmark startup state and will occur when either the first shear half cycle has not yet been completed or when the Landmark Controller has power cycled and will remain until the Landmark Controller has all the navigation data required to compute valid RPC values.

- A sequence number of -2 indicates that that there is no valid data available. This is typically due to the unavailability of data from the shearer-mounted navigation system.

- A sequence number of -3 indicates that the longwall operator has not enabled (or temporarily disabled) the face alignment system and therefore no RPCs can be issued.

- The Roof Support system requests Face Profile values by setting a Face Profile Required flag (RSS Class 0x64, Attribute 9 – Status bit 1).

- The face profile functionality is identical to the above RPC functionality

Class 0x4 Instance 1: Face Adjustment

- Following the sequence number is an array of INTs, one for each roof support module present.

- Each member of the array is a Recommended Position Correction (RPC) in mm. Each member is an integer, so no fraction of mm is possible

- RPCs are negative valued with a maximum value of 0 and describe how much less than full stroke each support should advance. For example an RPC value of zero will result in the support advancing full stroke.

Class 0x4 Instance 2: Face Profile

- Following the sequence number is an array of INTs, one corresponding to each roof support module present.

- Each member of the array is a Face Profile value in mm. Each member is an integer, so no fraction of mm is possible. These values describe the horizontal profile of the face at points corresponding to support.

- These values are normalized so the first (maingate end) value is always zero.

Class 0x4 Instance 3: Roof Support Ram Extension

- This assembly object provides instantaneous D/A ram extension information to the client.

- The internal update rate of this data is controlled by the Roof Support server and no explicit strategy for determining data freshness has been included. The internal data update period should not exceed 30 seconds.

Instance 4: Roof Support Leg Pressure

- This assembly object provides leg pressure information to the client. Provision is made for up to four pressure transducers per roof support. Transducers 1 & 2 are used for two legged supports.

- Values corresponding to unused or uninstalled transducers should be set to zero.

- The internal update rate of this data is controlled by the Roof Support server and no explicit strategy for determining data freshness has been included. The internal data update period should not exceed 30 seconds.

Class Attributes:

- Check class 0x64 attrib 1 =1 (EDS) revision

- Check class 0x64 attrib 3 = 4 (fixed) number of instances

- Check class 0x64 attrib 8 = (EDS) default advance distance

- Check class 0x64 attrib 13 =  (EDS) panel width

- Check class 0x64 attrib 14 =  (EDS) gate width

# 4 Testing Extension Details

There are no requirements to extend the basic testing application (core_redirect and eip_redirect) functionality added by this specification.

# 5 Testing Details

Each test will provide:

- Component identification (requirement number)

- Test summary

- Testing input requirements

- Test function

- Testing output requirements

- Fail Severity

| Test case | Test pre-requisites | Input State/Requirements | Event | Output | Fail Severity |
| --- | --- | --- | --- | --- | --- |
| - Functional Testing |  |  |  |  |  |
| - Register EIP Session | NIL | Idle Device | Register EIP Session | Successful if EIP Session Registered | Critical |
| - Heartbeat | 1.1 | EIP Session Registered | Send 1 heartbeat, and wait for correct response. | Fails if device does not respond to heartbeat correctly | High |
| - 16 x Heartbeats | 1.2 | EIP Session Registered | Send 16 sequential heartbeats with 500ms delay between each | Fails if device does not respond to 16 heartbeats correctly | High |
| - Get Number of Roof Supports | 1.3 | EIP Session Registered | Get number of roof supports from RSS | Fails if device does not respond with number of roof supports | Medium |
| - Check Basic Face Profiles |  | EIP Session Registered | Send sequence -1 (invalid sequence number) | Successful if face profile displayed on client DOES NOT match the one sent | High |
| - Check Basic Face Profiles |  | EIP Session Registered | Send sequence -2 (invalid sequence number) | Successful if face profile displayed on client does NOT match the one sent | High |
| - Check Basic Face Profiles |  | EIP Session Registered | Send valid sequence number | Successful if face profile displayed on client matches the one sent | High |
| - Check Basic Face Profiles |  | EIP Session Registered | Send 2nd valid sequence number | Successful if face profile displayed on client matches the one sent | High |
| - Check Basic Face Profiles |  | EIP Session Registered | Send repeated sequence number (invalid) | Successful if face profile displayed on client DOES NOT match the one sent | High |
| - Check Basic RPC | 1.10 | EIP Session Registered | Send sequence -1 (invalid sequence number) | Successful if RPC displayed on client DOES NOT match the one sent | High |
| - Check Basic RPC Seq No -1 |  | EIP Session Registered | Send sequence -2 (invalid sequence number) | Successful if RPC displayed on client does NOT match the one sent | High |
| - Check Basic RPC Seq No -2 |  | EIP Session Registered | Send valid sequence number | Successful if RPC displayed on client matches the one sent | High |
| - Check Basic RPC Seq No -3 |  | EIP Session Registered | Send 2nd valid sequence number | Successful if RPC displayed on client matches the one sent | High |
| - Check Basic RPC Transmission Pattern |  | EIP Session Registered | Send repeated sequence number (invalid) | Successful if face profile displayed on client DOES NOT match the one sent | High |
| - Check Roof Support Ram Request |  | EIP Session Registered |  | Successful if Roof Support Ram extension profile matches the one displayed | Medium |
| - Leg and Set Transmission |  | EIP Session Registered |  | Successful if Roof Support Leg and Set Pressure profiles match the ones displayed | Medium |
| - Class Attributes |  |  |  |  |  |
| - Class 0x64 Attribute 1 | 1 | EIP Session Registered | Get Revision | Successful if RSS returns valid response | Medium |
| - Class 0x64 Attribute 3 | 2.1 | EIP Session Registered | Get Number of Instances | Successful if RSS returns valid response | Medium |
| - Class 0x64 Attribute 8 | 2.2 | EIP Session Registered | Get Default Advance Distance | Successful if RSS returns valid response | Medium |
| - Class 0x64 Attribute 9 | 2.3 | EIP Session Registered | Get Status | Successful if RSS returns valid response | Medium |
| - Class 0x64 Attribute 13 | 2.4 | EIP Session Registered | Get Panel Width | Successful if RSS returns valid response | Medium |
| - Class 0x64 Attribute 14 | 2.5 | EIP Session Registered | Get Gate Width | Successful if RSS returns valid response | Medium |