LASC Level 2 Compliance

OEM Accessible SPMS Data Functional Testing

Conformance Process for Specification and Acceptance Testing of LASC Communication Protocol

| VERSION: [1.0] | REVISION DATE: [7/10/09] |
| --- | --- |

| Approver Name | Title | Signature | Date |
| --- | --- | --- | --- |

Table of Contents

1. Introduction	3

2 Functional Requirements	3

3 Communications Requirements	3

4 Testing Extension Details	4

5 Testing Details	4

# 1. Introduction

This testing regime covers the encoding of complete testing of LASC Level2 Compliance for an SPMS compliant device.

Design document ‘Interconnection of Landmark Compliant Longwall Mining Equipment – Shearer Communication Specification for OEM Accessible SPMS Data.’ should be read in conjunction with this document.

The objective of this Component of the Landmark project is to provide instantaneous and diagnostic data from an SPMS or SPMS compliant device. The instantaneous roll, pitch and status data is used by an OEM to provide highly accurate, synchronous body pose information. This accurate and stable data can be used by the OEM control system as an alternative or in addition to independent shearer inclinometers. The diagnostic information is used to monitor system status and health.

# 2 Functional Requirements

- The SPMS device will respond to EIP requests at a maximum of 5Hz

- The SPMS will provide a status flag when the navigation data is reliable

- The SPMS will provide system health and diagnostic information

- The SPMS will not directly control any devices on the network, but only provide information to any client that connects to the SPMS

- Once calibrated, the SPMS will provide pitch and roll information at accuracy in the order than 1/100th degree.

# 3 Communications Requirements

Instance 1: OEM accessible inertial sensor data

- The system status value (Attribute 8, Instance 1) will be used as a heartbeat

- When the system is aligned, the status value will have bit 0 set to 1, and bit 1 set to 1

- The SPMS will return the correct Roll and Pitch information when requested, at a maximum of 5 Hz

Instance 2: OEM accessible system diagnostic data

- Attribute 1 will return the time since the system was last powered up

- Attribute 2 will return the current voltages available on the system

- Attribute 3 will return the Overvolts flags

- Attribute 4 will return the current relay states in the MUPS

- Attribute 5 will return the current temperature at the internal CPU temperature sensor

- Attribute 6 will return the current odometry provided to the SPMS system

- Attribute 7 will provide the status of  all the components of the system in a struct.

- Attribute 8 will provide the IP addresses of the system components

- Attribute 9 will provide the current CPU code version.

# 4 Testing Extension Details

There are no requirements to extend the basic testing application (core_redirect and eip_redirect) functionality added by this specification.

# 5 Testing Details

Each test will provide:

- Component identification (requirement number)

- Test summary

- Testing input requirements

- Test function

- Testing output requirements

- Fail Severity

| Test case | Test pre-requisites | Input State/Requirements | Event | Output | Fail Severity |
| --- | --- | --- | --- | --- | --- |
| - Functional Testing |  |  |  |  |  |
| - Register EIP Session | NIL | Idle Device | Register EIP Session | Successful if EIP Session Registered | High |
| - Heartbeat | 1.1 | EIP Session Registered | Send 1 heartbeat, and wait for correct response. | Fails if device does not respond to heartbeat correctly | High |
| - 16 x Heartbeats | 1.2 | EIP Session Registered | Send 16 Concurrent heartbeats with 200ms delay between each | Fails if device does not respond to 16 heartbeats correctly | High |
| - Check accuracy - Roll |  | EIP Session Registered |  | Successful if device responds with accurate roll information | High |
| - Check accuracy - Pitch |  | EIP Session Registered |  | Successful if device responds with accurate pitch information | High |
| - Class Attributes |  |  |  |  |  |
| - Class 0x73 Attribute 1 | 1 | EIP Session Registered | Get Revision | Successful if SPMS returns valid response | Medium |
| - Instance Attributes |  |  |  |  |  |
| - Class 0x73 Instance 1 Attribute 8 |  | EIP Session Registered | Get Status | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 1 Attribute 9 |  | EIP Session Registered | Get Pitch | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 1 Attribute 10 |  | EIP Session Registered | Get roll | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 2 Attribute 1 |  | EIP Session Registered | Get system up time | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 2 Attribute 2 |  | EIP Session Registered | Get System voltages | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 2 Attribute 3 |  | EIP Session Registered | Get System Overvolts flag | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 2 Attribute 4 |  | EIP Session Registered | Get System relay states | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 2 Attribute 5 |  | EIP Session Registered | Get System Temperature | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 2 Attribute 6 |  | EIP Session Registered | Get System Odometry | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 2 Attribute 7 |  | EIP Session Registered | Get Unit Status | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 2 Attribute 8 |  | EIP Session Registered | Get IP addresses | Successful if SPMS returns valid response | Medium |
| - Class 0x73 Instance 2 Attribute 9 |  | EIP Session Registered | Get CPU code version | Successful if SPMS returns valid response | Medium |